Table 1. Examples of analog and analog-digital SAs, belonging to different groups

| Name of semicustom array | Manufacturer/author | Technology/ Type | Remarks |  |  |
| --- | --- | --- | --- | --- | --- |
| G e n e r a l a p p l i c a t i o n S A s |  |  |  |  |  |
| STKM2000SERIES, STKM4000SERIES | STMicroelectronics | BiCMOS/ analog digital | Characterized cells - ADC, DAC,comparators, oscillators, operational amplifiers, bandgap, power-on-reset |  |  |
| The 700 Series Chips | Array Design Inc. | bipolar/analog | Chip Series 710…747: 33*33…118*119 mils, 18…630 transistors. |  |  |
| An Analog-Digital CMOS Semicustom Array for High-Performance Mixed-Mode ASICs | Shlemin D.L. Siberian State University of Telecommunications and Information Sciences. Novosibirsk, Russia | CMOS/ analog digital | The functional ability - up to 42 op amps, more than 250 pF and almost 2 Mohm of overall capacitance and resistance respectively and 1500 logic gates. The array is suitable for high-performance SC circuit design, though, other circuit techniques such as SI and continuous-time analog signal processing are possible. |  |  |
| Start-1, Start-2 | SRIPEA, Zelenograd, Russia | bipolar/analog | Number of pins 24/32, frequency band 0…100 МHz. Application fields: sensor, automotive, communication electronics, environment monitoring, power engineering. |  |  |
| Gennum Bipolar Linear Array (GA911) | Canadian Microelectronics Corporation | bipolar/analog | It consists of 112 active components and 88 passive components: 28 small NPN transistors, 12 small PNP transistors, 28 P- diffused resistors, 8 200-Ohm P+ diffused resistors, 4 90-kOhm pinch resistors,. |  |  |
| # БМК 2ФМ | Тechnological center, Zelenograd, Russia | bipolar/analog | The БМК 2ФМ contains: npn transistors with FT>800МHz, β>200; pnp vertical transistors with FT>100МHz, β>80; diodes, resistors Rs=200 Оhm/square; pinch-resistors; chip size 2.3x3.0mm sq. Number of leads - 24 The operating frequency band of functional units, built on the basis of БМК 2ФМ extends up to 100MHz |  |  |
| AS17xx | Astec | bipolar/analog | Basic Electrical Specs • Transistor Matching (NPN & PNP) <2% • Primary voltage limitations: LVCEO 18 V BVCBO 30 V | Diffusion to substrate (Ground) 30 V • NPN Parameters Beta 80–500 fT (1mA) 300MHz | BVEBO 7 V • PNP Parameters: Beta 20–300 fT (1mA) 300MHz BVEBO 30 V |

| F u n c t i o n a l l y o r i e n t e d S A s |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Field Programmable Analog Array (FPAA) AN10E40 | Anadigm | CMOS/ analog |  | Features of AN10E40 - 20 Programmable Analog Cells - 13 Analog IO Cells - 2 Spare Op-Amps - 8 Bit Programmable Internal Vref Source | - 4 Programmable Internal Clock Sources - Easy Power-On-Reset Self Boot Using Serial PROM - Microprocessor Boot Option - Intuitive Design Software Drift Free Designs |  |
|  | Mask Programmable Analog Array | LEM NCPHEP, Minsk, Belorussia | --/analog |  | Mask programmable analog array chip is 2.7x3.6 mm2 in size. One 100 mm wafer can hold up to 650 chips. Manufactured ICs: micropower low-noise charge sensitive amplifier, oscilloscope synchronization IC |  |  |
|  | ispPAC (In-System Programmable Analog Circuit):ispPAC10, ispPAC20 | Lattice Semiconductor | --/analog (+DAC) |  | FUNCTIONAL CELLS: instrumentation amplifier, output amplifier, 2,5-volt reference source, 8-digit DAC with voltage output and dual comparator. Joining several cells allows to build tunable active filters, based on the use of integrative cells, for the band of (10…100)kHz. |  |  |
|  | TRAC - Field Programmable Analog Devices | Fast Analog Solution (Zetex Semiconductors) | --/analog |  | Configurable analog cell, soft programmed to perform different analog functions: ADDITION NEGATE LOG ANTILOG AMPLIFY. TRAC020LHQ36: 20 analog cells, Small signal bandwidth 12 MHz. |  |  |
|  | Ap p l i c a t i o n s p e c i f i c S A s |  |  |  |  |  |  |
|  | Analog bipolar SA | Dvornikov О., Minsk, Belorussia | JFET – bipolar/analog | Intended to design multichannel ICs for nuclear electronics. Characteristics of npn transistors: Ft > 3GHz, Rbb < 35Ohm, – of p-JFETs: Up = (2…3)V, Ft>120MHz |  |  |  |
|  | Application specific SA (ASSA) | Cooperated design of MEPhI (contents and structure of chip) and Research Institute“Pulsar”, Moscow, (technology) | NPN bipolar/analog | General characteristics of ASSA: - unity-gain frequency of low-power microwave npn transistors – about 7GHz; - total number of transistors – over 1300; - total number of passive elements (diffus. resistors and MOS-capacitors) – over 5600; - total capacitance of MOS-capacitors – close to 200pF; - chip size – 6.25.5mm sq; number of contact pads – 58; |  |  |  |
|  | Complementary analog SA (ASA) | Cooperated design of MEPhI and SRIPEA | bipolar/analog | see section 3 |  |  |  |